Updating Firmware on uSDX+ – KC1SLE

Updating Firmware on uSDX+

March 3, 2024 by Trevan

I recently acquired a used –and seemingly a little faulty– uSDX+. These are the little SDR transceivers all over eBay based on the tr(uSDX) devleopment and produced by a plethora of vendors of varying quality. I believe mine is what is called the ‘white buttons’ model — in reference to the different variations that turn up– this has white buttons.

The one I have seems to have some hardware faults that give it almost too much noise to hear a signal over. Somewhere I came across a page the suggested that some of the firmware used on these is very outdated and could benefit from updating. There’s a process documented elsewhere but almost always involves using an Arduino Nano or similar as a programmer.

I thought I’d share the process I used instead in case anyone else is hoping to bypass the Arduino as programmer as and just use an existing programmer they may have. I didn’t find anyone who had done this. It’s actually a fairly straightforward process, but as with anything DIY, I am not responsible if this process does not work for you or causes damage to your device.

- Locate the firmware you want to use – in my case, I went with the latest release of uSDXOpen for my “white buttons” model. Simply download the .ino file for your model.

- Install (if you don’t already have) the Arduino IDE. You can now open the .ino file with the Arduino IDE and read the code and the various notes included. *Read it carefully.* There are notes about removing a capacitor from the radio and disabling the internal mic before programming otherwise the MCU may be damaged.

- Open up the top cover on the uSDX and locate the JTAG pins

- Locate a suitable programmer – I used a cheap USBasp programmer from Amazon.

- Connect up the programmer to the JTAG pins, making sure to get the direction correct. (My radio included no indication of which pins were which and I had to try twice to get it right- thankfully no damage occured from my error.)

- Plug in the programmer to your USB on your computer. In my case, at this time the radio turned on and made very loud noise.

- Download, install and open the program Zadig (a driver utility).

- Under Options, select List All Devices.

- Select “USBasp” (or your programmer) from the list of USB devices then, under target driver select “linusb(K)”. Then click ‘Replace Driver’

- Once that completes, open up the .ino file you downloaded with the Arduino IDE.

- Read through the code and make necessary configurations (setting your call sign as desired and various functional options). Once you’re happy, under Tools, select Boards > Amtel atmega328p Xplained mini. If you do not have this board, you may need to install it under Boards Manager.

- Then Tools > Programmer > USBasp (or your programmer)

- Then go to Sketch > Upload Using Progammer. The code will then compile and if successful begin uploading. You may also receive one or more errors you will need to investigate, including a problem with the size of the code (you’ll need to shrink it by disabling unwanted features), trouble starting the programmer (if the drivers or somethign else is wrong), or a number of other warnings or errors.

- If successfully completed, your radio can be disconnected and closed up.

- I recommend on the first boot to do a full reset — in my case this was done by holding the first button (Menu) during power on.

- Enjoy your new firmware!

Unfortunately, in my case, the firmware did not solve the noise issue- so I’ll be tinkering a bit more in the future. Looking at the PCB, there are numerous missing components from the board based on the silk screen, so I suspect one or more of these missing parts may account for the poor performance….