Step 1: Get the firmware file

Download the latest .bin firmware file from:
https://github.com/skuep/AIOC/releases

Step 2: Put the AIOC into bootloader/DFU mode

If this is the AIOC's very first flash (never updated before):

- Unplug the AIOC from USB

- Short (bridge) the two outermost pins on the programming header with a jumper, tweezers, or similar

- While still shorted, plug the USB cable back in — this puts it into bootloader mode

- Windows will detect it as "STM32 BOOTLOADER"

If the AIOC already has firmware version 1.2.0 or newer:

- You can skip the pin-shorting step entirely — newer firmware lets you trigger DFU mode purely through software during the flash command itself

Step 3 (Windows only): Install a libusb-compatible driver

dfu-util needs libusb access, and Windows' default "STM32 BOOTLOADER" driver won't work with it directly.

- Download Zadig: https://zadig.akeo.ie

- Run Zadig with the AIOC plugged in and in bootloader mode

- In the device dropdown, select the STM32 BOOTLOADER device

- Next to the driver selection arrow, choose WinUSB

- Click Install Driver and wait (can take ~30 seconds) — it'll confirm success

(Linux/macOS users can skip this step entirely — dfu-util works out of the box once libusb is installed.)

Step 4: Install dfu-util

- Windows: Download from https://dfu-util.sourceforge.net/ and extract the archive

- Linux/macOS: Install via your package manager, e.g.:

sudo apt install dfu-util      # Debian/Ubuntu

brew install dfu-util          # macOS with Homebrew

Step 5: Run the flash command

From the folder containing both dfu-util and your downloaded .bin firmware file:

Windows:

dfu-util.exe -a 0 -s 0x08000000:leave -D aioc-fw-x-y-z.bin

For NA6D specifically:

dfu-util.exe -d 1209:7388 -a 0 -s 0x08000000:leave -D aioc-fw-x-y-z.bin

Linux/macOS:

dfu-util -a 0 -s 0x08000000:leave -D aioc-fw-x-y-z.bin

Replace aioc-fw-x-y-z.bin with the actual filename you downloaded (e.g., aioc-fw-1.4.0.bin).

Notes

- On Linux/macOS, if you get a permissions error, either run with sudo or add your user to the plugdev group.

- After a successful flash, the AIOC will automatically reboot into the new firmware — no extra reset step needed.

- If something goes wrong and the device seems unresponsive afterward, redo the pin-shorting bootloader-entry step from Step 2 and try again.

Let me know what OS you're actually running this from and I can narrow these down further if any step trips you up.